## 🏢 Escenario: CaribeTech — Nueva sucursal

CaribeTech tiene ahora tres routers que conectan:

- Headquarters

- Branch Office

- Data Center

La empresa quiere dejar de mantener rutas estáticas manualmente y utilizar OSPF.

El objetivo de esta laboratorio es construir una red OSPF Area 0 completamente funcional.

🗺️ Topología

# 🎯 Objetivos

## Parte 1 — Switching

En SW1 — 3560 L3:

Configura:

- VLAN 10

  - vlan 10: Create the vlan 10

  - name IT: Name the vlan

- VLAN 20

  - vlan 20: Create the vlan 20

  - name sales: Name the vlan

- SVIs

  - Vlan 10

    - Interface Vlan 10: Create vlan interface

    - ip add 10.10.10.254 255.255.255.0: Assign the IP to the interface

  - Vlan 20

    - Interface Vlan 20: Create vlan interface

    - ip add 10.10.20.254 255.255.255.0: Assign the IP to the interface

- Inter-VLAN routing

  - ip routing

  - Interface f0/1: Enter port FastEthernet 0/1

  - switchport mode acc: Change port to access mode

  - switchport acc vlan 10: Assign port to the vlan 10.

  - Interface f0/2: Enter port FastEthernet 0/2

  - switchport mode acc: Change port to access mode

  - switchport acc vlan 20: Assign port to the vlan 20.

En SW2 — 2960:

Configura:

- VLAN 30

  - Vlan 30: Creates vlan 30

  - Name HR: Assign a name to the vlan

- Access ports

  - Interface range f0/1-2: Enter ports FastEthernet 0/1 and 0/2

  - switchport mode acc: Change ports to access mode

  - switchport acc vlan 30: Assign port to the vlan 30.

- Uplink hacia R2

  - Interface g0/0: Enter interface GigabitEthernet 0/0

  - switchport mode trunk: Change the port to a trunking state

  - switchport trunk allowed vlan 30: Allow traffic tag with vlan 30 to pass through.

## Parte 2 — Routing

On Switch 1:

- ip routing: Put the switch on a layer 3 mode

- int g 0/0: Enters the interfaces

- no switchport: Changes the switch interface to a Layer 3

- ip add 10.255.0.14 255.255.255.252: Assing IP address to the interface

- no sh: turn the interface up.

On R1:

- int g 0/0: Enters the interfaces

- ip add 10.10.30.254 255.255.255.252: Assing IP address to the interface

- no sh: turn the interface up.

Repeat the process until all needed interfaces have an assigned IP address.

## Parte 3 — OSPF

Configura OSPF single-area.

Todos los routers deben participar en: Area 0

Debes anunciar:

- Redes de HQ:

  - On Router 2:

    - router ospf 1: Enables ospf

    - Network 10.10.10. 0.0.0.255(/24) area 0: Announced the network

    - Network 10.10.30.0 0.0.0.255(/24) area 0: Announced the network

- Redes de Branch

  - On Switch 1:

    - router ospf 1: Enables ospf

    - Network 10.10.20.0. 0.0.0.255(/24) area 0: Announced the network

    - Network 10.10.10.0.0. 0.0.0.255(/24) area 0: Announced the network

- Red del Data Center:

  - On Router 3:

    - router ospf 1: Enables ospf

    - Network 10.10.40. 0.0.0.255(/24) area 0: Announced the network

- Redes WAN

  - On Router 1:

    - router ospf 1: Enables ospf

    - Network 10.255.0.0 0.0.0.3(/30) area 0: Announced the network

    - Network 10.255.0.4 0.0.0.3(/30) area 0: Announced the network

  - On Router 2:

    - router ospf 1: Enables ospf

    - Network 10.255.0.0 0.0.0.3(/30) area 0: Announced the network

    - Network 10.255.0.8 0.0.0.3(/30) area 0: Announced the network

    - Network 10.255.0.12 0.0.0.3(/30) area 0: Announced the network

  - On Router 3:

    - router ospf 1: Enables ospf

    - Network 10.255.0.4 0.0.0.3(/30) area 0: Announced the network

    - Network 10.255.0.8 0.0.0.3(/30) area 0: Announced the network

  - On Switch 1:

    - router ospf 1: Enables ospf

    - Network 10.255.0.12 0.0.0.3(/30) area 0: Announced the network

Note: This configuration needs to be done on every router and the Layer 3 Switch

# OSPF Challenge #1

Configurar Router IDs manualmente:

On Router 1:

- router-id 1.1.1.1

On Router 2:

- router-id 2.2.2.2

And like that until all routers participating on OSPF have a router id.

# OSPF Troubleshooting Challenge

Ahora viene una parte importante.

Una vez que todo funcione, desconecta físicamente el enlace: R1 - R2

Después revisa:

show ip route

show ip ospf neighbor

# OSPF Challenge #2 — Cost

Ahora quiero que experimentes con OSPF cost.

Haz que el enlace:  R1 ↔ R3 sea menos preferido que: R1 ↔ R2

- int g0/1: Enters the interface

- ip ospf cost 10: Give the cost to the route

Después utiliza:

show ip route

y:

traceroute

para comprobar qué camino selecciona OSPF.

Luego rompe el enlace preferido y observa qué sucede.

# Additional commands:

- Passive-interface G0/0: Make the interfaces passive and do not try to make unnecessary ospf neighbors.